---
jupytext:
[[space2]]formats: md:myst
[[space2]]text_representation:
[[space4]]extension: .md
[[space4]]format_name: myst
kernelspec:
[[space2]]display_name: Python 3
[[space2]]language: python
[[space2]]name: python3
editor_options: 
[[space2]]markdown:
[[space4]]wrap: none
---
(i_cam_equipment)=
# {{ title_i_cam_equipment }}
::::::::{hint}
:::{seealso}
{bdg-link-primary-line}`Camera settings<https://ab-rcsc.github.io/rc-decision-support-tool_concept-library/02_dialog-boxes/01_40_cam_settings.html>`
:::

::::::::

::::::::{tab-set}

:::::::{tab-item} Overview
## Camera hardware options

Remote cameras consist of a digital camera with a lens, external flash, and a passive infrared and/or motion detector (among other features; [Figure 4](#TOC_surv_guidelines_fig_4)).

:::{figure} ../0_figures/Survey-guidelines_CamFeatures.png
:align: center
:scale: 60%
:name: TOC_surv_guidelines_fig_4
:::

**Figure 4.** Examples of the a) external components and b) internal controls and components of a remote camera (Reconyx PC900; Reconyx Inc., 2017)

The camera "make" is the manufacturer of a particular camera (e.g., Reconyx), and the "model" is the model number of a particular camera (e.g., PC900). There are many different options and features to choose from when deciding upon the best {{ cam_make_tu }} and {{ cam_model_tu }} for a particular study, which differ in their impacts on {{ detection_probability_tl_abrv }}. For this reason, deploying multiple [Camera Models](#camera_model) within a study is not advisable (Palencia et al., 2022; Wellington et al., 2014).

It is common for new camera users to confuse the specifications of a particular {{ cam_make_tu }} and {{ cam_model_tu }} with the camera's settings. Specifications refer to the camera's features (characteristics), while settings are options that the user can change. When choosing a {{ cam_make_tu }} and {{ cam_model_tu }}, important specifications include {{ trigger_speed_tl }}, {{ recovery_time_tl }}, {{ detection_zone_tl }} (i.e., the area [conical in shape] in which a remote camera can detect the heat signature and motion of an object Rovero & Zimmermann, 2016; see [Figure 5](#TOC_surv_guidelines_fig_5)), battery life and flash type. The best choice of {{ cam_model_tu }} will depend on the {{ survey_objectives_tu_pl }}, {{ mod_approach_tl }}, {{ target_species_tu }}, and physical environment.

Here are a few examples of specifications to achieve certain {{ survey_objectives_tu_pl }}:

-   To estimate {{ density_tl }} with the [random encounter models (REM;](#mods_rem) {{ density_tl }}) approach -- use a camera with a fast {{ trigger_speed_tl }}, a wide {{ detection_zone_tl }}, no-glow infrared (IR) flash, and the ability to take bursts of photos (Rovero et al., 2013).

-   To estimate {{ density_tl }} or abundance with mark-recapture methods -- use a camera with a white flash, a short {{ recovery_time_tl }}, and a fast {{ trigger_speed_tl }} (Rovero et al., 2013). Note that white flashes may scare some animals and potentially reduce the number of recaptures (Séquin et al., 2003; Wegge et al., 2004).

-   [Occupancy studies](#mods_occupancy) need a fast {{ trigger_speed_tl }} ({{ settings_trigger_sensitivity_tu }} - high) (although the importance of which is species-dependent; Rovero et al., 2013).

-   Faunal detections generally require a fast {{ trigger_speed_tl }} ({{ settings_trigger_sensitivity_tu }} - high) and a wide {{ detection_zone_tl }}.
::: {note}
Most [modelling approaches](#mod_approach) require a fast {{ trigger_speed_tl }} (however, the use of {{ baitlure_bait_tl }} or {{ baitlure_lure_tl }} may compensate for slower [trigger speeds](#trigger_speed) in some cases).
:::

Given the numerous [Camera Models](#camera_model) available and the frequent release of new models, it would be difficult to recommend a make and model to fit all users' needs. However, there are many studies and reviews that compare the specifications and the utility of different [Camera Models](#camera_model) (e.g., see <https://www.trailcampro.com/collections/trail-camera-reviews>; <https://www.mammalweb.org/images/schools/Camera-trap-buying-guide.pdf>; Fisher & Burton, 2012; Rovero et al., 2014; Rovero & Zimmermann, 2016; Seccombe, 2017; Wearn & Glover-Kapfer, 2017).

:::{figure} ../0_figures/Survey-guidelines_FOV_DetZone.png
:align: center
:scale: 60%
:name: TOC_surv_guidelines_fig_5
:::

**Figure 5.** The ability to detect an animal will vary according to the camera specifications (and settings). Important specifications include the camera's detection zone (here termed "trigger area"), Field of View (FOV; "viewable area'), and"registration area" (the area in which an animal entering has at least some probability of being captured on the image) (Moeller et al., 2023).

(TOC_surv_guidelines_battery_type)=
### 7.1.1 Battery type

Most remote cameras require AA batteries. It is recommended to use **lithium batteries,** as opposed to alkaline or nickel metal hydride, because they are less affected by cold temperatures. Battery life will be affected not only by the type of batteries but also by the camera settings, temperature, and number of images or videos taken (which are dependent on the camera settings, placement, and level of activity in front of the camera) (Wearn & Glover-Kapfer, 2017). However, some camera user manuals contain information on battery performance and the total number of images a camera can be expected to collect before the batteries die (based on the operating temperature and battery type, e.g., [Reconyx HyperFire Instruction Manual](https://images.reconyx.com/file/HyperFireManual.pdf) [Reconyx Inc., 2017]).

(TOC_surv_guidelines_sd_cards)=
### 7.1.2 SD cards

It is important to consider the **size**, **type**, and **class** of **SD (Secure Digital) card** since the available options vary in storage capacity, compatibility, and write-speed (Wearn & Glover-Kapfer, 2017).

The **size of the SD card** (i.e., storage capacity) should be considered in relation to the expected duration of {{ deployment_tl }}, the deployment area, and the level of activity expected to occur in front of the camera. For example, a camera placed in a grassy area might be expected to produce more {{ false_trigger_tl_pl }} due to grass waving in front of the camera. Or perhaps a camera placed near a den might be expected to have higher animal activity. Both situations might warrant using a larger SD card. A 4 GB SD card is capable of storing \~8,000-20,000 images (400-900 KB in size), which might be sufficient if you plan to revisit the camera frequently (\~every four weeks) (Wearn & Glover-Kapfer, 2017). We suggest using a card with at least 16 GB, and Wearn & Glover-Kapfer (2017) suggest larger (32 GB) if the video is enabled or if the camera will be active for long periods.

There are three **types of SD card:** standard (SD; maximum memory of 2 GB), high-capacity (SDHC; maximum memory of 32 GB) and extended-capacity (SDXC; maximum memory of \> 32 GB) (Wearn & Glover-Kapfer, 2017). Note that SDHC cards are not compatible with most [Camera Models](#camera_model). Be sure to check the camera user manual to confirm the compatible SD card type(s) (Wearn & Glover-Kapfer, 2017).

The **“class” of an SD card** (e.g., class 2, 4, 6, or 10) indicates the “write-speed” (i.e., the speed at which the SD card can read and write data; Wearn & Glover-Kapfer, 2017). Slower write speeds may perform poorly if the camera is set to collect images continuously, as fast as possible (i.e., rapid-fire or “near-video”) or if the video setting is activated. It is recommended to use an SD card of class 4 or higher, ideally class 10 (Wearn & Glover-Kapfer, 2017).

Caution should be used when deploying older SD cards (Wearn & Glover-Kapfer, 2017) and, perhaps, microSD card types; a few remote camera users in Alberta have described a 50% SD card failure rate with microSD cards (St. Clair, personal communications). See Wearn & Glover-Kapfer (2017) for additional information on choosing and maintaining (i.e., regularly formatting) SD cards.
:::::::

:::::::{tab-item} In-depth
```{include} include/00_coming_soon.md
```
:::::::

:::::::{tab-item} Visual resources
::::::{grid} 3
:gutter: 3
:class-container: wrapper

:::::{grid-item-card} {{ rtxt_rcsc_et_al_2024 }}
<img src="../03_images/03_image_files/rcsc_et_al_2024_CamFeatures.png" class="img_grid"><br><br>   
:::::

:::::{grid-item-card} {{ rtxt_wearn_gloverkapfer_2017 }}
<img src="../03_images/03_image_files/wearn_gloverkapfer_2017_fig4_3_clipped.png" class="img_grid"><br><br>   
:::::

:::::{grid-item-card} {{ rtxt_figure3_ref_id }}
<img src="../03_images/03_image_files/model_reconyxpc900_hf_pir.png" class="img_grid"><br><br>   
:::::

:::::{grid-item-card} {{ rtxt_figure4_ref_id }}
<img src="../03_images/03_image_files/model_reconyxhp2x.png" class="img_grid"><br><br>   
:::::

:::::{grid-item-card} {{ rtxt_figure5_ref_id }}
<img src="../03_images/03_image_files/model_bushnellx_8.png" class="img_grid"><br><br>   
:::::

:::::{grid-item-card} {{ rtxt_wearn_gloverkapfer_2017 }}
<img src="../03_images/03_image_files/wearn_gloverkapfer_2017_fig10_1.png" class="img_grid"><br><br>   
:::::

:::::{grid-item-card}
<img src="../03_images/03_image_files/Error! Reference source not found." class="img_grid"><br><br>Error! Reference source not found.
:::::

:::::{grid-item-card}
<img src="../03_images/03_image_files/Error! Reference source not found." class="img_grid"><br><br>Error! Reference source not found.
:::::

:::::{grid-item-card}
<img src="../03_images/03_image_files/Error! Reference source not found." class="img_grid"><br><br>Error! Reference source not found.
:::::

:::::{grid-item-card}
<img src="../03_images/03_image_files/Error! Reference source not found." class="img_grid"><br><br>Error! Reference source not found.
:::::

:::::{grid-item-card}
<img src="../03_images/03_image_files/Error! Reference source not found." class="img_grid"><br><br>Error! Reference source not found.
:::::

:::::{grid-item-card}
<img src="../03_images/03_image_files/Error! Reference source not found." class="img_grid"><br><br>Error! Reference source not found.
:::::

:::::{grid-item-card} {{ rtxt_vid1_ref_id }}
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="vid1_url"></iframe></div>

:::::

:::::{grid-item-card} {{ rtxt_vid2_ref_id }} 
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="vid2_url"></iframe></div>

:::::

:::::{grid-item-card} {{ rtxt_vid3_ref_id }}
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="vid3_url"></iframe></div>

:::::

:::::{grid-item-card} {{ rtxt_vid4_ref_id }} 
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="vid4_url"></iframe></div>

:::::

:::::{grid-item-card} {{ rtxt_vid5_ref_id }}
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="vid5_url"></iframe></div>

:::::

:::::{grid-item-card} {{ rtxt_vid6_ref_id }}
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="vid6_url"></iframe></div>

:::::

:::::{grid-item-card}
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="Error! Reference source not found."></iframe></div>

Error! Reference source not found.
:::::

:::::{grid-item-card}
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="Error! Reference source not found."></iframe></div>

Error! Reference source not found.
:::::

:::::{grid-item-card}
<div class="iframe-container-vid"><iframe class="iframe-responsive-vid" src="Error! Reference source not found."></iframe></div>

Error! Reference source not found.
:::::

::::::

:::::::

:::::::{tab-item} Shiny apps/Widgets
Check back in the future!
:::::::

:::::::{tab-item} Shiny apps/Widgets
::::::{card} shiny_name
shiny_caption

<div class="iframe-container-shiny"><iframe class="iframe-responsive-shiny" src="shiny_url"></iframe></div> 
::::::

:::::::

:::::::{tab-item} Shiny apps/Widgets
::::::{card}
:::::{dropdown} shiny_name
shiny_caption

<div class="iframe-container-shiny"><iframe class="iframe-responsive-shiny" src="shiny_url"></iframe></div> 
:::::

:::::{dropdown} Error! Reference source not found.
Error! Reference source not found.

<div class="iframe-container-shiny"><iframe class="iframe-responsive-shiny" src="Error! Reference source not found."></iframe></div> 
:::::

::::::

:::::::

:::::::{tab-item} Analytical tools & Resources
| Type | Name | Note | URL | Reference |
|:----------------|:-------------------------------|:----------------------------------------------------------------|:----------------------|:----------------------------------------|
|
|
|
|
|
|
 
:::::::

:::::::{tab-item} References
  Refs

:::::::

::::::::